1. Which of the following best describes pipelining in computer architecture?
A) Using multiple CPUs to execute multiple programs simultaneously.
B) Overlapping execution of instructions to improve throughput.
C) Storing frequently used data for quick access.
D) Using cache memory to speed up processing.

2. Which is NOT a type of pipeline hazard?
A) Data hazard
B) Control hazard
C) Structural hazard
D) Storage hazard

3. What is a pipeline stage?
A) A register that stores data permanently.
B) A step in the instruction execution broken into phases.
C) The main memory where data is fetched from.
D) An ALU that performs multiplication only.

4. Which technique helps to handle data hazards?
A) Prefetching
B) Branch prediction
C) Forwarding (bypassing)
D) Virtual memory

5. Which of these hazards is caused by branch instructions?
A) Data hazard
B) Structural hazard
C) Control hazard
D) Resource hazard

6. The speedup from pipelining is ideally equal to:
A) The clock cycle time
B) The number of pipeline stages
C) The instruction length
D) The number of processors

7. What does ‘pipeline stall’ mean?
A) The pipeline is fully utilized.
B) Instructions are executed out of order.
C) The pipeline stops due to a hazard.
D) There is no branch prediction.

8. Which of the following is true about pipeline performance?
A) Pipeline increases the execution time of a single instruction.
B) Pipeline always increases latency.
C) Pipeline reduces throughput.
D) Pipeline eliminates all hazards automatically.

9. Which unit detects hazards and stalls the pipeline if needed?
A) ALU
B) Control unit
C) Cache controller
D) Branch predictor

10. Which of these is a common pipeline depth in modern CPUs?
A) 1–2 stages
B) 3–4 stages
C) 5–20 stages
D) 100+ stages

11. Numerical: A non-pipelined processor takes 100 ns per instruction. If pipelined into 5 stages with each stage taking 20 ns, what is the ideal speedup?

12. Which factor limits maximum pipeline depth?
A) Instruction length
B) Clock skew and latch delay
C) ALU size
D) Bus width

13. Numerical: If a pipeline has 5 stages and executes 100 instructions ideally, how many clock cycles will it take?

14. Which of these reduces control hazards?
A) Cache memory
B) Branch prediction
C) Forwarding unit
D) Virtual memory

15. What is pipeline throughput?
A) Time to execute one instruction
B) Number of instructions executed per unit time
C) Total time for fetching
D) The instruction length divided by clock cycle

16. What happens if there is a structural hazard in a pipeline?
A) No impact on performance
B) Two instructions compete for the same resource
C) Clock speed increases
D) Branch prediction is not needed

17. Numerical: A pipelined processor has a cycle time of 5 ns and 5 stages. If there are pipeline stalls causing a 20% performance loss, what is its actual speedup compared to a non-pipelined time of 25 ns per instruction?

18. Which is true about instruction latency in pipelining?
A) It always increases
B) It always decreases
C) It stays the same
D) It has no effect

19. Numerical: What is the minimum number of pipeline stages needed to achieve a speedup of 4 under ideal conditions?

20. Numerical: If a pipeline has a clock cycle of 2 ns and executes 500 instructions in 503 cycles, what is the throughput in MIPS?

21. Numerical: A processor executes each instruction in 80 ns without pipelining. If pipelined into 4 stages with 25 ns per stage, what is the ideal speedup?

22. Which type of hazard occurs if two instructions need the same hardware resource at the same time?
A) Data hazard
B) Control hazard
C) Structural hazard
D) None of these

23. Numerical: If a pipeline has 6 stages and executes 200 instructions ideally, how many clock cycles will it take?

24. Which technique is commonly used to reduce branch penalties?
A) Loop unrolling
B) Caching
C) Branch prediction
D) Pipelining

25. Numerical: A pipeline has a cycle time of 10 ns. If there is a 10% stall penalty, what is the effective cycle time per instruction for an ideal 5-stage pipeline?

26. Which of these is not a benefit of pipelining?
A) Increased throughput
B) Reduced instruction latency
C) Better CPU utilization
D) Overlapping execution

27. Numerical: A program has 100 instructions. Without pipelining, it takes 500 ns per instruction. If pipelined into 5 stages with a 110 ns cycle per stage, how long will it take to execute all instructions ideally?

28. Which hazard occurs due to dependencies between instructions?
A) Structural hazard
B) Control hazard
C) Data hazard
D) All of the above

29. What is the main reason for using pipeline registers between stages?
A) Store final program output
B) Hold intermediate data between stages
C) Reduce memory size
D) Execute more branches

30. Numerical: A 4-stage pipeline has a cycle time of 5 ns. How many instructions can it execute in 500 ns under ideal conditions?

31. What is ‘pipeline depth’?
A) The number of registers in a stage
B) The number of stages in the pipeline
C) The length of instruction queue
D) The memory capacity

32. Which hazard is the hardest to handle automatically?
A) Structural hazard
B) Data hazard
C) Control hazard
D) None — all are easy

33. Which instruction causes the most pipeline flushes?
A) Arithmetic
B) Load/store
C) Branch
D) I/O

34. Which method does not solve data hazards?
A) Forwarding
B) Pipeline stalling
C) Branch prediction
D) Instruction reordering

35. What is the main drawback of deeper pipelines?
A) Higher branch penalties
B) Less throughput
C) Larger instruction size
D) Smaller caches

✅ Answers — Pipelining MCQs & Numericals

1. B → Overlapping execution improves throughput — the essence of pipelining.
2. D → No such thing as “storage hazard” — the 3 hazards are data, control, structural.
3. B → Each stage = phase of instruction execution.
4. C → Forwarding/bypassing reduces data hazards.
5. C → Branches create control hazards.
6. B → Ideal speedup = number of stages.
7. C → A stall means the pipeline stops temporarily.
8. A → Pipelining improves throughput, but the execution time of each instruction stays roughly the same — so latency per instruction may not reduce.
9. B → The control unit handles hazard detection and stalls.
10. C → Modern CPUs have 5–20 pipeline stages.

✅ Numericals & Conceptual

11. Speedup = Non-pipelined time / Pipeline stage time
= 100 ns / 20 ns = 5

12. B → Clock skew, latch delay limit max stages.
13. Ideal: K + (N – 1) cycles → 5 + (100–1) = 104 cycles
14. B → Branch prediction reduces control hazards.
15. B → Throughput = instructions per unit time.
16. B → Structural hazard = resource conflict.
17. Ideal speedup = 25 ns / 5 ns = 5
20% loss → Actual = 5 × 0.8 = 4
18. A → Latency per instruction often increases because it passes through more stages.
19. Stages = ideal speedup → So, minimum = 4 stages.
20.
Time per instr = (503 cycles × 2 ns) / 500 = ~2.012 ns per instr
MIPS = 1 / (2.012 ns) × 10⁻³ = ~497 MIPS → ~497 MIPS
21. Speedup = Non-pipeline time / pipeline stage time = 80 ns / 25 ns = 3.2 (not full 4 due to stage time)
22. C → Structural hazard = resource conflict.
23. 6 + (200–1) = 205 cycles
24. C → Branch prediction reduces branch penalties.
25. Ideal CPI = 1 for pipeline → 10 ns per instr.
With 10% stalls: 10% extra → Effective = 11 ns.
26. B → Latency usually increases, so reduced latency is NOT a benefit.
27. Ideal cycles: K + (N–1) = 5 + (100–1) = 104 cycles
Total time = 104 × 110 ns = 11,440 ns
28. C → Data hazard = dependency.
29. B → Registers hold intermediate values.
30. Ideal: instructions = (time – filling)
Time: 500 ns
Filling = K–1 = 3 cycles × 5 ns = 15 ns
Time for steady state: 485 ns → Throughput = 1 instr per cycle = 5 ns/instr → 485/5 = 97 instr
Plus filling: 1 filling cycle for first instr → total = 97 + 1 = 98 instructions
31. B → Depth = number of stages.
32. C → Control hazards (branches) are hardest to handle automatically.
33. C → Branch instructions cause flushes.
34. C → Branch prediction does not solve data hazards — it’s for control hazards.
35. A → Deeper pipelines = higher branch misprediction penalties.

1. Which of the following is NOT a basic part of a CPU?
A) Control Unit
B) Arithmetic Logic Unit
C) Cache Memory
D) Keyboard

2. The part of the CPU that performs arithmetic and logical operations is called:
A) Control Unit
B) Register
C) ALU
D) Bus

3. The part of the CPU that fetches and decodes instructions is:
A) Arithmetic Logic Unit
B) Control Unit
C) Register
D) RAM

4. The high-speed temporary storage inside the CPU is called:
A) RAM
B) Hard Disk
C) Register
D) CD-ROM

5. The main function of the Control Unit is to:
A) Perform calculations
B) Store data permanently
C) Control and coordinate CPU operations
D) Display output

6. Which component stores frequently used instructions for quick access?
A) RAM
B) ROM
C) Cache
D) Hard Disk

7. Which of the following best defines clock speed?
A) The number of instructions per second
B) The number of cycles per second
C) The storage capacity of the CPU
D) The number of cores in the CPU

8. The clock speed is measured in:
A) GB
B) MB
C) Hz
D) RPM

9. 1 GHz equals how many cycles per second?
A) 1,000 cycles
B) 1 million cycles
C) 1 billion cycles
D) 1 trillion cycles

10. If a CPU has a clock speed of 2.5 GHz, how many cycles does it perform in 1 second?
A) 2.5 million
B) 2.5 billion
C) 2,500
D) 2.5 trillion

11. Which of these can limit CPU performance despite high clock speed?
A) Large cache
B) Better ALU
C) Thermal limits and heat
D) More registers

12. What is the role of registers in the CPU?
A) Permanent storage
B) Long-term storage
C) Temporary, fast storage
D) External storage

13. Which of these improves CPU performance?
A) Increasing clock speed
B) Adding more cores
C) Increasing cache memory
D) All of the above

14. If the CPU clock speed is doubled, what ideally happens to the processing speed?
A) It is halved
B) It stays the same
C) It doubles
D) It decreases

15. Which CPU part acts as a ‘traffic controller’?
A) ALU
B) Cache
C) Control Unit
D) RAM

🔢 Numerical & Conceptual Short Answer

16. A CPU runs at 3.2 GHz. How many clock cycles will it perform in 0.5 seconds?

17. If an instruction takes 4 clock cycles to complete and the CPU runs at 2 GHz, how many instructions can be executed in one second?

18. If a CPU can execute 1 instruction per clock cycle and runs at 4 GHz, what is the maximum instructions per second?

19. A CPU with 3.5 GHz clock speed executes a program with 10 million instructions. If the CPI (cycles per instruction) is 2, how much time will it take approximately?

| Q# | Answer |
| --- | --- |
| 1 | D |
| 2 | C |
| 3 | B |
| 4 | C |
| 5 | C |
| 6 | C |
| 7 | B |
| 8 | C |
| 9 | C |
| 10 | B |
| 11 | C |
| 12 | C |
| 13 | D |
| 14 | C |
| 15 | C |

Numerical:

- 3.2 GHz = 3.2 × 10⁹ cycles/sec
In 0.5 sec: 3.2 × 10⁹ × 0.5 = 1.6 × 10⁹ cycles

- 2 GHz = 2 × 10⁹ cycles/sec
Each instruction takes 4 cycles → 2 × 10⁹ / 4 = 500 million instructions/sec

- 4 GHz = 4 × 10⁹ instructions/sec (since 1 instruction per cycle)

- 3.5 GHz = 3.5 × 10⁹ cycles/sec
Program: 10 million instructions, CPI = 2
Total cycles = 10 million × 2 = 20 million
Execution time = Total cycles / Clock rate = 20 × 10⁶ / 3.5 × 10⁹ ≈ 0.0057 sec

1️⃣ A CPU has a clock speed of 2.8 GHz. How many cycles does it complete in 2 seconds?

2️⃣ A program requires 1 billion instructions to execute. If the CPU clock speed is 3.0 GHz and the average CPI is 3, how long will it take to run the program?

3️⃣ If a CPU runs at 2.5 GHz and executes 4 instructions per clock cycle (superscalar), how many instructions can it execute per second?

4️⃣ A CPU runs at 4 GHz. If an instruction needs 5 clock cycles, how many instructions can it complete per second?

5️⃣ A CPU can run at either 2.0 GHz or 3.5 GHz. A program needs 50 million instructions with a CPI of 2. Find the execution time for both speeds.

6️⃣ A computer has a CPU clock rate of 1.8 GHz. A benchmark program takes 1.5 seconds to execute. If the CPU completes 900 million instructions during this time, find the average CPI.

7️⃣ A CPU with a 3.2 GHz clock speed completes a task in 0.4 seconds. If the task used 2 billion cycles, what is the average CPI if the task had 400 million instructions?

8️⃣ A CPU executes a program in 0.8 seconds at 2.4 GHz with an average CPI of 1.5. How many instructions did it execute?

9️⃣ If you increase the clock speed from 2.0 GHz to 2.5 GHz, what is the percentage increase in clock cycles per second?

10️⃣ A CPU with a 5 GHz clock speed executes 10 million instructions with an average CPI of 4. What is the execution time in milliseconds?

1️⃣ 5.6 billion cycles
2️⃣ 1 second
3️⃣ 10 billion instructions per second
4️⃣ 800 million instructions per second
5️⃣ 0.05 sec (2.0 GHz)
  0.0286 sec (3.5 GHz)
6️⃣ CPI = 3
7️⃣ CPI = 5
8️⃣ 1.28 billion instructions
9️⃣ 25% increase
🔟 8 milliseconds

Q1. Which of the following correctly represents the typical order in a memory hierarchy from fastest to slowest?
A) Main Memory → Registers → Cache → Secondary Storage
B) Registers → Cache → Main Memory → Secondary Storage
C) Cache → Registers → Main Memory → Secondary Storage
D) Secondary Storage → Main Memory → Cache → Registers

Answer: B) Registers → Cache → Main Memory → Secondary Storage

Q2. The primary reason for using a memory hierarchy is:
A) To increase processing speed only
B) To reduce cost only
C) To balance cost, speed, and capacity
D) To make programming easier

Answer: C) To balance cost, speed, and capacity

Q3. Which level of the memory hierarchy typically has the largest capacity?
A) Registers
B) Cache
C) Main Memory (RAM)
D) Secondary Storage

Answer: D) Secondary Storage

Q4. Which of these is usually implemented directly inside the CPU?
A) RAM
B) Cache
C) Hard Disk
D) Magnetic Tape

Answer: B) Cache

Q5. The hit ratio of a cache is defined as:
A) Number of cache misses divided by total memory accesses
B) Number of cache hits divided by total memory accesses
C) Time taken to access the cache
D) Size of the cache memory

Answer: B) Number of cache hits divided by total memory accesses

Q6. Which of the following is NOT a type of cache mapping technique?
A) Direct Mapping
B) Fully Associative Mapping
C) Hybrid Mapping
D) Set-Associative Mapping

Answer: C) Hybrid Mapping

Q7. When the CPU requests data that is not present in the cache, it is called a:
A) Cache hit
B) Cache miss
C) Page fault
D) Segmentation fault

Answer: B) Cache miss

Q8. Which memory has the shortest access time?
A) Hard Disk
B) RAM
C) Cache
D) Register

Answer: D) Register

Q9. Which of the following statements is true about locality of reference?
A) Only spatial locality exists in programs
B) Only temporal locality exists in programs
C) Programs do not exhibit any locality
D) Programs tend to access a relatively small portion of their address space at any instant

Answer: D) Programs tend to access a relatively small portion of their address space at any instant

Q10. Which storage type is volatile?
A) Hard Disk
B) SSD
C) RAM
D) Blu-ray Disc

Answer: C) RAM

2️⃣ Memory Addressing MCQs

Q1. In a computer system with 16-bit addresses, what is the maximum addressable memory size?
A) 16 bytes
B) 64 KB
C) 32 KB
D) 256 KB

Answer: B) 64 KB

Q2. Which addressing mode uses the address part of the instruction to specify the operand directly?
A) Immediate addressing
B) Direct addressing
C) Indirect addressing
D) Register addressing

Answer: B) Direct addressing

Q3. In immediate addressing mode, the operand is:
A) Stored in memory
B) Stored in a register
C) Given explicitly in the instruction
D) Found using a pointer

Answer: C) Given explicitly in the instruction

Q4. Which addressing mode is used when an instruction specifies the register containing the operand?
A) Direct
B) Register
C) Indirect
D) Indexed

Answer: B) Register

Q5. Which of the following is NOT an addressing mode?
A) Direct
B) Indexed
C) Displacement
D) Assignment

Answer: D) Assignment

Q6. If the base register holds 1000H and the index register holds 0020H, what is the effective address in base-indexed addressing?
A) 1020H
B) 1000H
C) 0020H
D) 1002H

Answer: A) 1020H
(Note: This addressing mode is not used in 8085.)

Q7. In 8085, the instruction MVI A, 05H uses which addressing mode?
A) Immediate
B) Direct
C) Indirect
D) Register

Answer: A) Immediate

Q8. In indirect addressing mode, the address field of the instruction contains:
A) The actual operand
B) The address of the operand
C) The index register
D) The opcode only

Answer: B) The address of the operand

Q9. Which addressing mode allows accessing array elements easily?
A) Direct
B) Immediate
C) Indexed
D) Register

Answer: C) Indexed
(Again, note: Indexed addressing is not used in 8085.)

Q10. If a processor has a 32-bit address bus, what is its maximum addressable memory?
A) 64 KB
B) 4 GB
C) 16 MB
D) 2 GB

Answer: B) 4 GB

A processor runs at 2.5 GHz and has a CPI (cycles per instruction) of 2. If a program has 1 billion instructions, how long will it take to execute?
A) 0.8 seconds
B) 0.5 seconds
C) 1 second
D) 0.4 seconds

Q2. A cache has a hit ratio of 95% and the cache access time is 5 ns. The main memory access time is 50 ns. What is the average memory access time (AMAT)?
A) 7.25 ns
B) 5.25 ns
C) 7.75 ns
D) 10 ns

Formula: AMAT = (Hit ratio × Cache time) + (Miss ratio × Main memory time)

Q3. A computer with a 32-bit address bus uses byte addressing. How much memory can it address?
A) 4 KB
B) 4 MB
C) 4 GB
D) 2 GB

Q4. If an instruction in 8085 takes 7 clock cycles and the clock frequency is 3 MHz, what is the time taken to execute that instruction?
A) 2.33 µs
B) 1.5 µs
C) 3 µs
D) 4 µs

🔢 Numerical MCQs on Memory Addressing

Q5. In a system with 16-bit address bus, what is the hexadecimal value of the highest memory location?
A) 0xFFFF
B) 0x10000
C) 0xFFFE
D) 0xFFFFF

Q6. In indirect addressing mode, if the instruction contains address 2000H and the memory location 2000H contains 3000H, which memory location will be accessed?
A) 2000H
B) 3000H
C) 5000H
D) 1000H

Q7. An array starts at memory location 2000H. If each element is 2 bytes and the processor uses indexed addressing, what is the address of the 5th element (indexing from zero)?
A) 2008H
B) 200AH
C) 2010H
D) 2004H

Q8. The 8085 microprocessor uses a 3 MHz clock frequency. How much time does it take for one clock cycle?
A) 0.33 µs
B) 3 µs
C) 0.3 ns
D) 33 µs